POLYTECHNIC UNIVERSITY OF THE PHILIPPINES

Bachelor of Science in Electronics Engineering (BSECE)

EXPERIMENT #1: SOLVING LINEAR EQUATIONS USING MATLAB

Complete Solutions & MATLAB Code

# PROBLEM 1: Gauss Elimination Method

System of equations:

x₁ + 2x₂ + 2x₃ + x₄ = 17

2x₁ − 4x₂ + x₃ − 5x₄ = 8

2x₁ + x₂ − 2x₃ − 4x₄ = 10

−x₁ + 2x₂ + x₃ − 2x₄ = 17

## Augmented Matrix Setup

[A|b] =

|  | x₁ | x₂ | x₃ | x₄ | b |
| --- | --- | --- | --- | --- | --- |
| Row 1 | 1 | 2 | 2 | 1 | 17 |
| Row 2 | 2 | -4 | 1 | -5 | 8 |
| Row 3 | 2 | 1 | -2 | -4 | 10 |
| Row 4 | -1 | 2 | 1 | -2 | 17 |

## Forward Elimination Steps

Step 1: Eliminate x₁ from rows 2, 3, 4 using Row 1 as pivot.

R2 = R2 - 2*R1   →   [0, -8, -3, -7 | -26]

R3 = R3 - 2*R1   →   [0, -3, -6, -6 | -24]

R4 = R4 + R1     →   [0, 4, 3, -1 | 34]

Step 2: Eliminate x₂ from rows 3, 4 using Row 2 as pivot.

R3 = R3 - (3/8)*R2  →   [0, 0, -4.875, -3.375 | -14.25]

R4 = R4 + (1/2)*R2  →   [0, 0, 1.5, -4.5 | 21]

Step 3: Eliminate x₃ from row 4 using Row 3.

R4 = R4 + (1.5/4.875)*R3  →   [0, 0, 0, -5.538 | 16.615]

## Back Substitution → Final Answer

| Variable | Value | Verification |
| --- | --- | --- |
| x₁ | 2.0000 | ✔ Satisfies all equations |
| x₂ | 4.0000 | ✔ Satisfies all equations |
| x₃ | 5.0000 | ✔ Satisfies all equations |
| x₄ | -3.0000 | ✔ Satisfies all equations |

# MATLAB Code (Problem 1)

% Problem 1 - Gauss Elimination

A = [1 2 2 1; 2 -4 1 -5; 2 1 -2 -4; -1 2 1 -2];

b = [17; 8; 10; 17];

% Method 1: MATLAB backslash operator

x = A \ b

% Method 2: Manual Gauss Elimination

Aug = [A b];

n = length(b);

for k = 1:n-1

for i = k+1:n

factor = Aug(i,k)/Aug(k,k);

Aug(i,:) = Aug(i,:) - factor*Aug(k,:);

end

end

x = zeros(n,1);

x(n) = Aug(n,n+1)/Aug(n,n);

for i = n-1:-1:1

x(i) = (Aug(i,n+1)-Aug(i,i+1:n)*x(i+1:n))/Aug(i,i);

end

disp('Solution x1, x2, x3, x4:'); disp(x')

# PROBLEM 2: Gauss-Jordan Elimination Method

System of equations:

5x₁ + 3x₂ + 2x₃ + x₄ = 27

2x₁ − 3x₂ + 4x₃ − 5x₄ = 40

x₁ + 2x₂ − 2x₃ − 4x₄ = 13

3x₁ + 4x₂ + 2x₃ + x₄ = 23

## Augmented Matrix Setup

|  | x₁ | x₂ | x₃ | x₄ | b |
| --- | --- | --- | --- | --- | --- |
| Row 1 | 5 | 3 | 2 | 1 | 27 |
| Row 2 | 2 | -3 | 4 | -5 | 40 |
| Row 3 | 1 | 2 | -2 | -4 | 13 |
| Row 4 | 3 | 4 | 2 | 1 | 23 |

## Gauss-Jordan Reduction to Diagonal Form

Unlike Gauss Elimination, Gauss-Jordan eliminates BOTH above and below each pivot, reducing the matrix to a diagonal (identity) form. No back substitution needed.

The process reduces [A|b] to [I|x] where I is the identity matrix:

| Step | Operation | Target Form |
| --- | --- | --- |
| 1 | Divide R1 by 5; eliminate x₁ from R2, R3, R4 | Pivot on column 1 |
| 2 | Normalize new R2; eliminate x₂ from R1, R3, R4 | Pivot on column 2 |
| 3 | Normalize new R3; eliminate x₃ from R1, R2, R4 | Pivot on column 3 |
| 4 | Normalize new R4; eliminate x₄ from R1, R2, R3 | Final diagonal form |

## Final Answer

| Variable | Value | Check |
| --- | --- | --- |
| x₁ | 3.0000 | ✔ |
| x₂ | 2.0000 | ✔ |
| x₃ | 5.0000 | ✔ |
| x₄ | -4.0000 | ✔ |

# MATLAB Code (Problem 2)

% Problem 2 - Gauss-Jordan Elimination

A = [5 3 2 1; 2 -3 4 -5; 1 2 -2 -4; 3 4 2 1];

b = [27; 40; 13; 23];

Aug = [A b];

n = length(b);

for k = 1:n

Aug(k,:) = Aug(k,:)/Aug(k,k); % Normalize pivot row

for i = 1:n

if i ~= k

Aug(i,:) = Aug(i,:) - Aug(i,k)*Aug(k,:);

end

end

end

x = Aug(:, n+1);

disp('Solution x1, x2, x3, x4:'); disp(x')

# PROBLEM 3: LU Decomposition (Gauss Elimination)

Circuit analysis using mesh current method. Four mesh loops are identified from the circuit diagram.

## KVL Mesh Equations

Applying KVL clockwise to each mesh loop:

Mesh 1 (i₁, top loop):   9i₁ − 4i₂ − 2i₃ + 0i₄ = 24

Mesh 2 (i₂, left loop):  −4i₁ + 17i₂ − 6i₃ − 3i₄ = 16

Mesh 3 (i₃, middle):    −2i₁ − 6i₂ + 14i₃ − 6i₄ = 0

Mesh 4 (i₄, right loop):  0i₁ − 3i₂ − 6i₃ + 11i₄ = 18

## Matrix Form [A][x] = [b]

|  | i₁ | i₂ | i₃ | i₄ | b |
| --- | --- | --- | --- | --- | --- |
| Row 1 | 9 | -4 | -2 | 0 | 24 |
| Row 2 | -4 | 17 | -6 | -3 | 16 |
| Row 3 | -2 | -6 | 14 | -6 | 0 |
| Row 4 | 0 | -3 | -6 | 11 | 18 |

## LU Decomposition Result

Using Doolittle's method: [A] = [L][U], where L is lower triangular with 1s on diagonal and U is upper triangular.

Lower Triangular Matrix [L]:

|  | Col 1 | Col 2 | Col 3 | Col 4 |
| --- | --- | --- | --- | --- |
| Row 1 | 1.0000 | 0 | 0 | 0 |
| Row 2 | -0.4444 | 1.0000 | 0 | 0 |
| Row 3 | -0.2222 | -0.4526 | 1.0000 | 0 |
| Row 4 | 0 | -0.1971 | -0.7049 | 1.0000 |

Upper Triangular Matrix [U]:

|  | Col 1 | Col 2 | Col 3 | Col 4 |
| --- | --- | --- | --- | --- |
| Row 1 | 9.0000 | -4.0000 | -2.0000 | 0 |
| Row 2 | 0 | 15.2222 | -6.8889 | -3.0000 |
| Row 3 | 0 | 0 | 10.4380 | -7.3577 |
| Row 4 | 0 | 0 | 0 | 5.2224 |

Verification: L × U = A ✔ (all residuals = 0)

## Two-Step Solution

Step 1: Forward substitution [L][y] = [b]:

| Variable | Value |
| --- | --- |
| y₁ | 24.0000 |
| y₂ | 26.6667 |
| y₃ | 17.4015 |
| y₄ | 35.5217 |

Step 2: Back substitution [U][x] = [y]:

| Current | Value (A) | Physical Meaning |
| --- | --- | --- |
| i₁ | 6.7766 A | Top mesh current |
| i₂ | 6.0166 A | Left mesh current |
| i₃ | 6.4617 A | Middle mesh current |
| i₄ | 6.8018 A | Right mesh current |

# MATLAB Code (Problem 3)

% Problem 3 - LU Decomposition

A = [9 -4 -2 0; -4 17 -6 -3; -2 -6 14 -6; 0 -3 -6 11];

b = [24; 16; 0; 18];

n = length(b);

% LU Decomposition (Doolittle's Method)

L = eye(n); U = zeros(n);

for i = 1:n

for k = i:n

U(i,k) = A(i,k) - L(i,1:i-1)*U(1:i-1,k);

end

for k = i+1:n

L(k,i) = (A(k,i) - L(k,1:i-1)*U(1:i-1,i))/U(i,i);

end

end

% Verify A = L*U

disp('A - L*U ='); disp(A - L*U)

% Step 1: Forward substitution Ly = b

y = zeros(n,1);

for i = 1:n

y(i) = b(i) - L(i,1:i-1)*y(1:i-1);

end

% Step 2: Back substitution Ux = y

x = zeros(n,1);

for i = n:-1:1

x(i) = (y(i) - U(i,i+1:n)*x(i+1:n))/U(i,i);

end

fprintf('i1=%.4f A, i2=%.4f A, i3=%.4f A, i4=%.4f A\n',x(1),x(2),x(3),x(4))

# VI. DISCUSSION QUESTIONS

## Q1: Differentiate the three direct methods (manual and program)

Gauss Elimination uses row operations to convert the augmented matrix into upper triangular form, then solves by back substitution. Manually this requires systematic elimination of coefficients below each pivot. In MATLAB, this is implemented by nested loops performing row operations on the augmented matrix.

Gauss-Jordan Elimination extends Gauss Elimination by eliminating coefficients both below AND above each pivot, reducing the matrix to diagonal (identity) form. The solution is read directly from the final augmented column with no back substitution needed. Manually it requires more operations but the final step is simpler. In MATLAB, an inner loop runs for all rows (not just rows below the pivot).

LU Decomposition factors the coefficient matrix A into a lower triangular L and upper triangular U such that A = LU. The system is then solved in two steps: forward substitution to solve Ly = b, then back substitution to solve Ux = y. Manually it involves computing L and U entries systematically. In MATLAB, this is done with nested loops for Doolittle's method, or simply using the built-in lu() function.

## Q2: Which method is more efficient? Why?

For a single system: Gauss Elimination is most efficient because it only processes below-pivot entries, requiring approximately n³/3 operations. Gauss-Jordan requires about n³/2 operations (50% more) since it also eliminates above-pivot elements.

LU Decomposition is most efficient when solving multiple systems with the same coefficient matrix A but different right-hand sides b. Once A is factored into LU (which costs ~n³/3), each new system requires only forward and back substitution (~n²). This makes it ideal for engineering applications where the circuit topology (A matrix) is fixed but different source values are tested.

In MATLAB specifically, the backslash operator (A\b) uses an optimized algorithm that is typically the most efficient option, as it automatically selects the best method based on matrix properties.

## Q3: Thoughts and Learnings

Marife:

This activity demonstrated how Linear Algebra concepts learned in theory are directly applied to engineering problems, particularly circuit analysis. Implementing these methods in MATLAB revealed how systematic and algorithmic solving can be, making previously tedious manual calculations fast and reliable.

Jo-ann:

I gained a deeper appreciation for how Gauss-Jordan and LU Decomposition are extensions and refinements of basic Gaussian elimination. Understanding the trade-offs between methods helps in choosing the right tool for different engineering scenarios.

Geoffrey:

Programming these algorithms in MATLAB reinforced the logic behind matrix operations. The LU decomposition activity was especially insightful because it showed how decomposing a problem into smaller sub-problems (forward then back substitution) is a powerful engineering strategy applicable beyond just linear algebra.

# VII. CONCLUSION

This experiment successfully demonstrated three direct methods for solving systems of linear equations: Gauss Elimination, Gauss-Jordan, and LU Decomposition. All three methods were implemented manually (via Excel) and programmatically (via MATLAB), yielding identical results, which validates both approaches.

For Problem 1 (Gauss Elimination), the solution was x₁ = 2, x₂ = 4, x₃ = 5, x₄ = −3. For Problem 2 (Gauss-Jordan), x₁ = 3, x₂ = 2, x₃ = 5, x₄ = −4. For Problem 3 (LU Decomposition circuit analysis), the mesh currents were i₁ = 6.78 A, i₂ = 6.02 A, i₃ = 6.46 A, and i₄ = 6.80 A, with the verification A = LU confirmed numerically.

MATLAB proved to be a powerful tool for numerical computation, reducing manual calculation time significantly while eliminating arithmetic errors. The activity reinforced the importance of linear algebra as a foundational tool in robotics and electronics engineering, particularly for circuit analysis and system modeling.